Plantlocation:Plantlocationisastrategicdecisionseveralfactorsinfluencethisdecision.Themainobjectiveofanybusinessistooptimizeitscostandrevenuethatis,minimizeitscostsandmaximizeitsreturns.

Thedegreeofsignificancefortheselectionoflocationforanyenterprisemainlydependsonitssizeandnaturelargescaleindustriesrequiringhugeamountofinvestmenttherearemanyconsiderationsotherthanthelocaldemandintheselectionproperplantlocationtheseplantscannotbeeasilyshiftedtootherplaceandanerrorofjudgmentintheselectionofsitecanbevaryexpensivetotheorganization.However,small-scaleindustrymainlyselectsthesitewhereinaccordancewithitscapacity;thelocalmarketisavailableforitsproducts.Itcaneasilyshifttootherplacewhenthereisanychangeinthemarket.

## Factorsaffectingplantlocation:

NearnesstoMarket:Iftheplantislocatedclosetothemarketthecostoftransportationcanbeminimized.Thisalsohelpstheproducerstohavedirectknowledgeoftherequirementsofthecustomers.

Nearnesstosupplyofrawmaterials:Asfaraspossiblethesiteselectedshouldbenearthesourceofrawmaterials,sothatthecostoftransportationcanbeminimizedandstoringcostcanbereducedduetoshorterleadtime.

Availabilityoflabour:Availabilityofrightkindoflabourforceinrequirednumberatreasonableratesisalsoadecidingfactorinselectionofsite

Transportand communicationfacilities:Generally, industries haveatendencytolocatetheindustrialunitsneartherailwaystation,highwayorportareas.

Availabilityofpowerandfuel:Coal,electricity,oilandnaturalgasaretheimportantsourcesofpowerintheindustries.

Ex:TataironandsteelindustryisestablishednearthecoalminesofBihar.Climaticconditions:	Climaticconditionslargelyaffect certainproductionprocessesandalsotheefficiencyoftheemployees.

Ex:TextilemillsrequiremoistclimatethatwhytheseplantlocatedatMumbaiandAhmedabad.

Availabilityofwater:Waterisusedinindustriesforprocessingasinpaperinchemicalindustries,forgenerationofpowerinhydroelectricpower,plantsandalsorequiredfordrinkingsanitarypurposealso.

Ancillaryindustries:Manyindustriessuchasprocessingandassemblyindustriesarenotproducingalthepartsoftheirproductbutpurchasesomeofthepartsfromancillaryindustriesproducingit.

Financialandotheraids:Forthedevelopmentofbackwardregionscentralaswellasstategovernmentprovidecertainincentivesandfacilitiessuchascash-subsides,concessionfinancialassistance,land,powerandotherfacilitiesatcheaperrates,taxconcessionetc.

PlantLayout:Atechniqueoflocatingmachines,processesandplantserviceswithinthefactoryinorder tosecurethegreatestpossibleoutputofhighqualityatthelowestpossibletotalcostofproduction

## Typeofplantlayout:

Productorlinelayout:  Thistypeoflayoutisdevelopedforproduct-focusedsystems.Inthistypeoflayoutonlyoneproduct,oronetypeofproduct,isproducedinagivenarea.Incaseof productbeingassembled,thistypeoflayoutispopularlyknownasanassemblylinelayout.

Theworkcentersareorganizedinthesequenceofappearance.Therawmaterialcentreatoneendofthelineandgoesfromoneoperationtoanotherrapidlywithminimumofwork-in-processstorageandmaterialhandling

Raw

Material

1

Cutting

2

3

4

5

6

7

Finished

Goods

TurningMilling	GrindingPaintingInspectpacking

ProcessorFunctionallayout:Thistypeoflayoutisdevelopedforprocessfocusedsystems.Theprocessingunitsareorganizedbyfunctionsintodepartmentsontheassumptionthatcertainskillsandfacilitiesareavailableineachdepartmentsimilarequipmentsandoperationsaregroupedtogether,e.g.,milling,foundry,drilling,plating,heattreatmentetc.

Theuseofprocess-focusedsystemsisverywideinbothmanufactureandotherservicefacilitiessuchashospitals,largeoffices,municipalservices,etc.

| RawMaterials | Foundry 1 | Turning 2 | Drilling 2 | Heattreatment 4 | Inspection 6 5 | FinishedGoods |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 Forging | 3 Milling | 4 3 Grinding | 5 Painting | 7 6 Packing |  |

Product-AProduct–B

Cellularorgrouplayout:Itisspecialtypeoffunctionallayoutinwhichthefacilitiesareclubbedtogetherintocells.Thisissuitableforsystemsdesignedtousetheconcepts,principlesandapproachesof‘grouptechnology’.Suchalayoutofferstheadvantagesofmassproductionwithhighdegreeofautomationevenifthenumbersofproductsaremorewithflexiblerequirement.Insuchasystemthe facilitiesaregroupintocellswhichareabletoperformsimilartypeoffunctionsforagroupofproducts.

Product-AProduct-B

Product-CProduct-D

LatheMilling

Shaper

Drilling

Drilling

Milling

Lathe

Milling

Drilling

Forging

GrindingBoring

Hardening

Testing

Inspection

Tempering

Packing

ABCD

JobShopLayout:Itisa  layoutforaverygeneralflexible  system thatisprocessing jobproduction,Thepreparationofsuchalayoutisdependentontheanalysisofthepossiblepopulationsofordersandisarelatively,complexaffair.

ProjectorFixedpositionLayout:Thisisthelayout forprojecttypesystemsinwhichthemajorcomponentiskeptatafixedpositionandallothermaterials,components,toolsmachines,worketc.arebroughtandassemblyorfabricationiscarriedout.Thistypeoflayoutisnownotusedverycommonlyasthemachinesrequiredformanufacturingworkarebigandcomplicated.Thefixedpositionlayoutisusedonlywhenitisdifficulttomovethemajorcomponentandfabricationistobecarriedout.Ex:productionofships.

Tools

HandlingEquipment

Machine–1

Machine–2

Main

Assembly

Worker–1

Worker–2

## Factorsinfluencingplantlayout:

Managementpolicy:Managementhastodecideonmanymatterse.g.natureandqualityofproducts,sizeoftheplant,integrationofproductionprocess,plansforexpansion,amountofinventoryinstock,employeefacilities

Manufacturingprocess:Thetypeofmanufacturingprocesse.g.synthetic/analytical,continuous/intermittentandrepetitive/non-repetitive,willgovernthetypeofplantlayout.

Natureofproduct:Smallandlightproductscanbemovedeasilytothemachines,whereasforheavyandbulkyproductsthemachinesmayhavetobemoved.

Typeofequipment:Theuseofsinglepurposeandmulti-purposemachinesubstantiallyaffectstheplantlayout.Similarly,noisyandvibratingmachinesrequirespecialattentionintheplantlayoutdecision.

Typesofbuildings:Theplantlayoutinasinglestoreybuildingwillbedifferentfromthatinamultistoreybuilding.Thecoveredareas,thenumberofstorey’s,elevatorsandstairs,parkingandstorageareaallaffectthelayout.

Availabilityoftotalfloorarea:Theallocationofspaceformachines,work-benches,sub-storeaislesetc.,ismadeonthebasisoftheavailablefloorareauseofoverheadspaceismadeincaseofshortageofspace.

Arrangementofmaterialshandingequipment:Providesufficientaislesforfreemovementofmaterialhandlingequipmentsuchashandtruck,forktrucketc.

Servicefacilities:Thelayoutoffactorymustincludeproperservicefacilitiesrequiredforthecomfortand welfareofworkers.Theseinclude canteen,lockers,drinkingwater,firstaidetc.

Possibilityoffutureexpansion:Plantlayoutismadeinthelightoffuturerequirementandinstallationsofadditionalactivities.

## Principlesofplantlayout:

Principleofintegration:Thebestlayoutisonewhichintegratesthemen,materials,machinery,supportingactivitiesandanyothersuchafactorsthatresultsinthebestcompromise.

Principleofminimummovement:Thenumberofmovementofworkersandmaterialsandthedistancemovedshouldbeminimized.Thematerialsshouldbetransportedinbulkratherthaninsmallamounts.

Principleofsmoothandcontinueflow:	Itstatesthatbottlenecks,congestionpointsandbulktrackingshouldberemovedbyproperlinebalancingtechniques.Principleofcubicspace: Spaceofaroom,ittheceilingheightisalsoutilized,morematerialscanbeaccommodatedinthesamespace.

Principleofsatisfactionofsafety:Workingplaces-safe,well-ventilatedandfreefromdust,noisefumes,odorsandotherhazardousconditions,helptoincreasetheefficiencyoftheworkersandimprovetheirmorale.

Principleofflexibility:Itmeansthebestlayoutinonewhichcanbeadoptedandre-arrangedataminimumcostwithleastinconvenience.

## Productivity:

Definition:Productivityisdefinedastherateatwhichthegoodsandservicesareproduced.

Itreferstotherelationshipbetweentheinputsandtheoutput.Itiscalculated asaratiobetween theamount producedandthe amountofresources(land,labour,capital,technologyetc.)usedinthecourseofproductioninotherwords

ProductivityOutput

Input

Andalsodefinedproductivityashumaneffortstoproducemoreandmorewithlessandlessinputsofresourcesasaresultofwhichthebenefitsofproductionaredistributedamongmaximumnumberofpeople.

## MethodofProduction:

Jobproduction:Inthissystem,goodsareproducedaccordingtotheorderswiththismethod,individualrequirementsoftheconsumerscanbemet.Eachjoborderstands aloneand is not likelyto be repeated.Thistypeof production hasalotofflexibilityofoperationandhencegeneralpurposemachinesarerequired.Factoriesadoptingthistypeofproduction,aregenerallysmallinsize.

Advantages:

- Itistheonlymethod,whichcanmeettheindividualrequirement.

- Thereisnomanagerialproblem,becauseofvery lessnumberofworkers,andsmallsizeofconcern.

- Suchtypeofproductionrequireslessmoneyandiseasytostart.Disadvantages:

- Thereisnoscopeforcontinuousproductionanddemand

- Asthepurchaseofrawmaterialsisless,hencecostofrawmaterialsperunitwillbeslightlymore.

- Forhandlingdifferenttypeofjobs,onlyskilledandintelligentworkersareneeded,thuslabourcostincreases.

Batchproduction:Thistypeofproductionisgenerallyadoptedinmediumsizeenterprise.Batchproductionisinbetweenjobproductionandmassproduction.Batchproductionisbiggerinscalethanthejobproduction.Whileitissmallerthanthatofmassproduction,batchproductionrequiresmoremachinesthanjobproductionandfewermachinesthattheofmassproduction.

Advantages:

- Whilecomparingwithmassproductionitrequireslesscapital

- Comparingwithjobproduction,itismoreadvantageouscommercially.

- Ifdemandforoneproductdecreasethenproduction,foranotherproductmaybeincreased,thustheriskoflossisveryless.

Disadvantages:

- Comparingwithmassproductioncostofscalesandadvertisementperunitismore

- Rawmaterialstobepurchasedareinlessquantitythanthatinmassproduction;thereforeitisslightlycostlierthanthatofmassproductionbecauselessquantitydiscountisavailable.

Massproduction:Thismethodofproductionisusedbyconcernswheremanufacturingiscarriedoncontinuouslyinanticipationofdemandthoughdemandoftheproductmaynotbeuniformthroughtheyear.

Inmassproduction,simplificationandstandardizationofproductsaremadewiththehelpofspecialized(onepurpose)machine,articlesofstandardizednaturecaneasilyandeconomicallybeproducedonalargescale.

Thereisasmalldifferencebetweenmassproductionandcontinuousproduction.Thisismainlyinthekindofproductanditsrelationtotheplant.Inmassproductionplantandequipmentareflexibleenoughtodealwithotherproducts,involvingsameproductionprocess.Whereasincontinuousorprocessproductiononlystandardizedproductinasequenceproduced.Inthismethodlayoutandrequirementofadditionaltoolsandequipment

Advantages:

- Asmoothflowofmaterialsfromoneworkstationtothenextinlogicalorder.

- Sincetheworkfromoneprocessisfeddirectlyintothenext,smallinprocessinventoriesresult

- Totalproductiontimeperunitshort

- Simpleproductionplanningcontrolsystemarepossible

- Littleskillisusuallyrequiredbyoperationsattheproductionline,hencetrainingissimple,shortandinexpensive.

Disadvantages:

- Abreakdownofonemachine may leadtoacompletestoppageofthelinethatfollowsthemachine.Hencemaintenanceandrepairischallengingjob.

- Sincetheproductdictatesthelayout,changesinproductdesignmayrequiremajorchangesinthelayout.

- Generallyhighinvestmentarerequiredowingtothespecializednatureofthemachinesandtheirpossibleduplicationintheline

WorkStudy:Workstudyisoneofthe mostimportantmanagementtechniqueswhichis employedtoimprove the activitiesintheproduction.Themain objectiveofworkstudyistoassistthemanagementintheoptimumuseofthehumanandmaterialresources.

Definition:Workstudyrefersto themethodstudyand work measurement, whichareusedtoexaminehumanworkinallitscontextsbysystematicallyinvestigating intoallfactorsaffectingitsefficiency andeconomytobring forththedesiredimprovement.

WorkMeasurementToassesshumaneffectiveness

MethodStudyToimprovemethodofproduction

WorkStudy

## MethodStudy:

Definition:Thesystematicrecordingandcriticalexaminationofexistingandproposed ways of doing work,asameansofdevelopingand applyingeasierandmoreeffectivemethodsandreducingcostitisalsocalledmotionstudy.

## WorkMeasurement:

Definition:Workmeasurementistheapplicationoftechniquesdesignedtoestablishtimeforaqualifiedworkertocarryoutaspecifiedjobatadefinedlevelofperformance.

Workstudyhastwoparts,MethodStudyandWorkMeasurement.Methodstudydealswiththetechniquesofanalyzingthewaytodoagivenjobbetter,WorkMeasurementseekstomeasurethetimerequiredtoperformthejob.

BasicprocedureforMethodStudy:Select: Theworktobestudied

Record:Alltherelevantfactsofthepresentorproposedmethodstudybyobservation

Examine:Therecordedfactscriticallyeverythingthatisdone,consideringinturn,thepurposeoftheactivity,theplacewhereitisperformed,thesequenceinwhichitisdone,thepersonwhoisdoingitandthemeansbywhichitisdone.

Develop:Themostpractical,economicalandeffectivemethodconsideringallthecircumstances.

Define:Thenewmethodsothatitcanalwaysbeidentified.Install:Themethodasstandardpractice

Maintain:Thatstandardpracticebyregularroutinechecks.

Recording:Thecurrentprocessofdoingthejobhastoberecorded,whiledoingsoeverydetailhoweversmallitmaybe,hastobeidentified.

Wherethe  process  is  too  long,involvingmanystages  ofproduction,inspectionortransportation,thepresentprocessofdoingthejobisrecordedsufficientlytogetherwithalltherelevantinformation,usingtheprocesschartsymbols.

| Symbol | Meanings |
| --- | --- |
|  | Operation:Operationinvolvingchangesintheconditionofaproduct Ex:Assemblyofspareparts |
|  | Transport:SomethingfromthelocationtoanotherEx:AssemblePCismovedtoinspectionsection |
|  | Storage:(permanent)Tostorethematerials,goodsetc.Ex:WhenPCisputintothestoreafterinspection |
|  | Delay:(Temporarystorage)Ariseswhentheproductwaitsfornextstageintheprocess Ex:Machinerybreakdownetc. |
|  | Inspection:Tocheckwhetherthequalityandquantityoftheproductissatisfactoryornot |
|  | Operation–cum–Inspection:Inspectionistakenplaceduringtheproductionprocess |
|  | Operation–cum–Transportation:Assembleistakingplacewhilethebeltconveyertransportsthespares. |

RecordingTechniques:Therecordingtechniquesareofthreetypes

- Processchart	b)Diagrams	c)Motionandfilmanalysisd)Models

## ProcessCharts:

    - Outlineprocesschart:	Thischartoutlinesthemaineventssequencewiseconsideringonlyoperationsandinspectionsinthegivenjob

Ex:	TASK	:ChangingrefillofaBallPointpenChartbegins:Unscrewthecap

Chartends	:ScrewthecapChartby	:--------------

ChartEnds	:--------------

1

2

3

4

5

6

1

7

UnscrewcapUnscrewneck

Removetheoldrefill

Assemblethespringonnewrefill

Placetherefillinthebarrel

| No.ofoperations | 7 |
| --- | --- |
| No.ofinspections | 1 |
| TotalNo.ofactivities | 8 |

Screwtheneck

Checkif theballpenwrites

Screwthecap

    - Flowprocesschart:Thesearescaledrawingsoftheworkplace,whichindicatewhereeachactivitytakesplace.Thischartiscapableofreflectingunduedelaysintransferringworkbetweenworkstationsduplicationofwork,andunfairworkassignment,whichmaydelaythecompletionprocess.Itclassifiedintothreetypes

ManType	:ItrecordswhattheworkerdoesMaterialsType	:ItrecordswhathappenstothematerialsEquipmentType	:Itrecordshowtheequipmentused.

      - ManType

## Ex:

Job	:Writingaletterusingshorthandtypist

Chartbegins	:Typistinownoffice-awaitingdictation

Chartends	:Typistputsletterandcopyinouttray

Typistoffice	:	6metersmanager’soffice

To author’s office	1

1	Takedictation

2	Toownoffice

        - PrepareTypeSet

        - TypeletterandcopyRemovefrommachine

        - andseparatecopy

1	Checkthecopy

- Placeinafileforsignature

| EVENT | NO | TIME | DISTANCE |
| --- | --- | --- | --- |
|  | 8 |  |  |
|  | 1 |  |  |
|  | 4 |  | 24meters |
|  | 1 |  |  |

Summary

To author’s office	3

1	Delayforsignature

4	Toownoffice

- Typeenvelopes

- Lettertoenvelopes

- Letterandcopyasidetoouttray

      - MaterialType:

Ex:	Job	:MakingthecastingreadyformachiningChartbegins:Castinglyinginfoundry

Chartends	:Castingreadyformachining

Activity

Operations

Distancemovedmts

Time

Remarks

Costinglayinginfoundrystore

-

-

-

Movedtogascuttingmachine

10

3

ByTrolley

Wait,cuttingmachinebeingset

-

5

-

Risescut

-

20

-

Waitfortrolley

-

10

-

Movedtoinspectiondepartment

6

2

-

Inspectionbeforemachining

-

15

ByTrolley

Movetomachineshop

10

3

-

## Summary

| Event | No. | Time | Distance |
| --- | --- | --- | --- |
|  | 1 | 20 | - |
|  | 1 | 15 | - |
|  | 1 | - | - |
|  | 2 | 15(5+10) | - |
|  | 3 | 8 | 26 |

activitiesofaworkershandarerecorded,intheirrelationshiptooneanother. Itiscommonlyusedforrepetitiveandshortoperations.

Ex:	Job	:AssembletowashersandnuttoboltChartbegins	:Handemptymaterialinboxes

PartNo.	:-------------

Chartends	:CompletedassemblyasidetoboxOperationNo.	:-------------

Method	:Present/proposed

Tobolt	1

Pickupbolt	1

ToPosition	2

ToBox	1

AsidetoBox	3

ToBolt	2

1	Tofirstwasher

1	Pickupwasher

2	Toposition

- Assembletobolt

- Tosecondwasher

3	Pickupwasher

4	Toposition

- Assembletobolt

- ToNut

5	PickupNut

- ToPosition

- Assembletobolt

1	Delay

Tofirstwasher

ofmore  thanoneitem(worker,machineor equipment)are recorded  on  acommontimescaletoshowtheirinterrelationship.

Byusingseparateverticalcolumnstorepresenttheactivitiesofdifferentoperatorsormachinesonacommontimescale,thechartshowsveryclearlytheperiodofidlenessonthepartofanyitemsduringtheprocess.

Ex:

ChartNo.-----------------SheetNo.-------------------Department---------------------

| Material | :B201casting | Job | :Makingaslotonthecasing |
| --- | --- | --- | --- |
| Machine Chartedby | :Slotted :------------ | Operation Date | :XYZ :------------ |

| Time(min.) | Man |  |  | Machine | Time(min.) |
| --- | --- | --- | --- | --- | --- |
| 0.2 | Removesfinishedcastingcleanswithcompressedair |  |  | Idle | 0.2 |
| 0.4 | Gaugesdepthofslotonsurfaceplate |  |  | Idle | 0.4 |
| 0.6 | Breakssharpedgeswithfilecleanswithcompressedair |  |  | Idle | 0.6 |
| 0.8 | Placeinaboxobtainsnewcasting |  |  | Idle | 0.8 |
| 1.0 | Cleansmachinewithcompressedair |  |  | Idle | 1.0 |
| 1.2 | Locatescastinginfixture,startsmachineandautomaticfee |  |  | Idle | 1.2 |
| 1.4 | Idle |  |  | Cuttingslot | 1.4 |
| 1.6 | Idle |  |  | Cuttingslot | 1.6 |
| 1.8 | Idle |  |  | Cuttingslot | 1.8 |
| 2.0 | Idle |  |  | Cuttingslot | 2.0 |

| Summary: Cycletime :2min. |  |  |
| --- | --- | --- |
| WorkingTime: a)Man :1.2min. | Utilization: a)Man | :60% |
| b)Machine :0.8min. Idletime: | b)Machine | :40% |
| - Man :0.8min. - Machine :1.2min. |  |  |

## Diagrams:

    - Flowdiagrams:Flowprocesschartonlyshowsthesequenceofvariousactivitiesnecessaryforperformingthespecifiedwork.Itdoesnotshowclearlythepathofmovementofmenandmaterialsfromonelocationtoanother.

Definition:Itisadiagrams, drawn toscale,intended to showthe relative positionoftheproductionmachineryandmarkstheroutefollowedbythemachines,materialsandmen.

    - Stringdiagrams:Theworkersaremovingatirregularintervalsbetweenanumberofpointsinaworkingareawithorwithoutmaterialsinmanyindustrialactivities.

Definition:Thestringdiagramisascaleplanormodelonwhichatreaditsusedtotraceandmeasurethepathofworkers,materialsorequipmentduringaspecifiedsequenceofevents.

    - Cyclegraph:Inthismethodasmallelectricbulbisattachedtoeachpartofthebody,whichmakesthemovementforcarryingoutanoperation.Thepathofmovementisphotographedbyhigh-speedcamera.

    - Chronocyclegraph:Thisisaphotographicrecord,whichtracesthepathofmovementontoaphotographicplace.Inprinciple,itissimilartothestringdiagram,andismosteffectivewhenrecordingshort,rapidmovements.

  - MicromotionStudyandfilmanalysis:Micromotionstudyisasetoftechniquesintendedtodividehumanactivityintogroupsofmovementsormicromotions (therbiligs) andthestudyofsuchmovements helptofindforan operatoronebestpatternofmovementsthatconsumeslesstimeandrequireslessefforttoaccomplishthetask.

Filmanalysis:Oncetheactivityhasbeenfilmedandthefilmprocessed,aprojectorrunsthefilmveryslowlyandthefilmcanbestoppedorreversedwheneverrequired.

    - SIMO Chart(simultaneousmotioncyclechart):ASIMOchartis basedon filmanalysis,itisagraphicrepresentationofthecoordinatedactivitiesofanoperatorsbodyembers.Theactivitiesaredescribedintermsofbasicorfundamentalmotions.Thetimerequiredforcompletionofthesemotionsisalsorecordedonthechart.

  - Models:Sometimesthepictureoftheexistingconditionsisnotclearbytheuseofflowprocesschartorflowdiagram.Insuchcasesinsteadofthescalesplansoftheshopfacilitiesmodelsareusedtoprovidevisualrepresentationoftheproposedlayoutbeforeproceedingwithactualrearrangementoftheworkplace.

WorkMeasurement:Workmeasurementistheapplicationoftechniquetoestablishthetimeforaqualifiedworkertocarryoutaspecifiedjobatadefinedlevelofperformance.

## ProcedureforWorkMeasurement:

    - Sect:Theworktobestudiedanddeterminetheobjectivesofthestudy

    - Record:Alltherelevantdatarelatingtocircumstancesinwhichtheworkisbeingdone,themethodstobeusedbreakdownthejobintoitselements

    - Examine:Therecordeddataandthedetailedbreakdowncriticallytoensurethemosteffectivemethodandmotionsarebeingusedandthatunproductiveelementsareseparatedfromproductiveelements.

    - Measure:Thetimerequiredtocompleteeachelementusingtheappropriateworkmeasurementtechniquesandcalculatethetimerequiredtocompetetheworkcyclewhichisknownasbasictime.

    - Compile:Thestandardtimefortheoperationorworkplace,incaseofstopwatchtimestudythevariousallowancestocoverrelation,personalneedsetc.areaddedtothebasictimetoestimatethestandardtime.

## Techniquesofworkmeasurement:

- Timestudy

- Synthesisfromstandarddata

- PredeterminedMotionTimeSystem(PMTS)

- Analyticalestimating

- WorkSampling

- Timestudy:Itisdefinedastheartofobservingandrecordingthetimerequiredtodoeachdetailedelementofallindustrialoperation.

Timestudyequipment:Timestudyequipmentcanbebroadlygroupedtwocategories

A)TimemeasuringdeviceB)Timestudyboardsandtimestudychart

- Timemeasuringdevices:

  - StopWatchb)Motionpicturecamerac)Timerecordingmachine

d)Electronictimer.a)StopWatch:

- Decimalminute stopwatch:In this type ofwatch themovementsisstartedandstoppedbymovingtheslide“A”,forwardandbackwardrespectivelyarecompleterevolutionoflargehandrepresents1minuteandsincethedialisdividedinto100partsreadingtowithin0.01minutescanbeobtained.Everytimethelargehandmakeonerevolutionthesmallhandwillregister1minuteandisabletoregisterupto30minutes.

- Decimalhourstopwatch:Thedialinthiswatchisdividedinto100parts.Theneedlecompletes10revolutionsinonehour.Theleastcountinthiswatchis

0.001hours.Thesmalldialofthiswatchisdividedinto30equlspaces(representing0.01hour)andthesmallneedlemakes31/3revolutionsinonehour.

- Motionpicture camera:Everyelementoftheoperationinvolvingmotionoftheworkersismadeintofilmthroughmotion picture camera when thisfilm is runataslowspeedthroughaprojector;thetimeofeachelementisrecordedusingastopwatch.

- Timerecordingmachine:Amovingtapeisrunisthismachineatauniformvelocityof10inches/minuteswiththehelpofelectricmotor.Themachinehastwokeys:onekey,whenpressed,indicatesstartingofanoperation,andtheotherkeyusedtotakeaprintonthescaledtapeattheendofelements.

- Electronictimer:Thetimingofstartingandendingofanoperationofanelementisautomaticallyrecordedthroughelectronictimers.

## B)Timestudyboardsandtimestudycharts:

Timestudyboard:Thesearesimpleandhandyhardwoodboardsequippedwithstopwatchholdersandclampsforholdingtheobservationsheetsandtimestudyforms.Theseboardshelptoseeandrecordtheobservationandtimeatthesameinstant.

Observationsforms:Printedorcyclostyledformsareusedforrecordingtheobservationduringthattimestudy.Itensuresthattimestudyaremadeina

standardmannerandthatnoessentialdataareomitted.Theseformsareattachedtothestudyboardbymeansofclipprovided

- Synthesisfromstandarddata:Thisonetechniqueofworkmeasurementtoobtainedsynthetictimesthataresynthesisfromelementtimespreviouslyobtainedfromdirecttime studies.Theanalysisandmeasurementstagearethusconductedpriortotheactualstudy.

- Predetermine motiontimesystem(PMTS):Everyelement ofworkiscomposedofsomecombinationofbasichumanmotions.Apartfrommentalactivityallworkscanbrokendownintoelementsthatusuallyafundamentalmovementofthebodyorbodymembers.Afterthisanalysisstagethebasicmotionsthathavebeenisolatedhaveatimeallottedtothemonthebasisofpredeterminemotiontimes.

- Analyticalestimation:Analyticalestimatingservesasbestformeasuringwork.IntheanalysisstagewefindtheusuallythesebasicelementsormuchlargerascomparedtotheelementsinPMTSortimestudy.Formeasuringstagesthetime,whichwillbeoccupiedbytheelementataspecificspeedofworkingisestimated.

- WorkSampling:Itisworkmeasurementtechniquewhichlargenumberofinstantaneousobservationaremaderandomintervaloveraspecifiedperiodoftimeofagroupofworkers,machineandprocesses.Eachobservationrecordswhatis happeningat thatinstantantandthe presentobservationsrecordedforaparticularactivityordelayisameasureofthepercentageoftimeduringwhichthatactivityordelayoccurs.

It can also definedas a methodoffindingthe percentageoccurrenceofacertainactivitybystatisticalsamplingandrandomobservations.Procedureforconductingtimestudy:	Forconductingtimestudyaverageworkersandaveragemachinesareselected.Thisstudyidconductedbythetimestudyexpert,whoshould befamiliarwithalltheinformation relatedto thejobandtheconditionsinwhichitisbeingdone.

Timestudyisperformedinthefollowingstages.

A)AnalysisofworkB)Standardizationofmethods

C)Makingtimestudy

- Analysisofwork:Itincludesallthetasksperformedbytheworkers,notjusttheeffectivework.Intheend,timerequiredforjobpreparation,cleaningofmachine,etc.shouldalsobeincluded.

- Standardizationofmethods:Relatedtomaterials,equipment,tools,workingconditionstoensureanacceptablemethodwhichiseasy,safeandthefastest.

- Makingtimestudy:Timestudyisdoneonaprintedtimestudyrecordsheet,whichisfixedonaboardknownastimestudyboard.Ononecorner,astopwatchisplaced.

Differenttimereadingsofelementare  recordedinthecorrespondingcolumnoftherecordsheet.Severalsetsofreadingaretakentoarriveatanaccurateresultafternotingallthesereadings,averagetomeiscalculated,neglectingabnormalvalues,ifany.

Standard time:it is thetime, whichis takenbya normal workerfora specifictaskorjob,workingundermoderateconditionsandincludingotherallowances.Suchasfatiguesettingoftoolandjob,repairingoftoolandcheckingofjobetc

Standard  timeisthebasisforthecalculationofwagesand

incentives.

Standardtime=Averagetime*Ratingfactor+otherallowances

Ratingfactor:thestudyengineermultipliesactualtimewithafactorknownasRatingfactororlevelingfactortosettheaveragetimewhichanormalworkerwouldtake.Thisisexpressedasapercentageoftheeffacingofrepresentativeoperator,whichisincomparisontosomeofhisaveragefellowworkers.

Performancerating:performanceratingisthatprocess,duringwhichthetimestudyengineercomparestheperformanceoftheoperatorofnormalperformance.

# Observedpersormance

Performancerating=		100Normalperformance

## Theratingcanbe

  - Standardrating:Aqualifiedworkerwillnaturallyworkifheismotivatedtoapplyhimselftohisworkattheaveragerateofpace.

  - Normalrating:Itistheaveragerateorpaceatwhichaqualifiedworkerwillnaturallyworkevenifhehasnospecificmotivationtoapplyhimselftothiswork.

## Ratingtechniques:

    - Speedrating:Speedratingconsistsofdeterminingthespeedoftheoperator’smovementsinrelationtoanormalpaceasafactorandapplyingittoeachelementsoastogetthenormaltimefortheelement.

# ObseredtimeRating

Normalorbasictime=

# Standardrating

Rating	=Worker’sspeed

Standardrating	=Speedexpectedfromtheworker

    - Westing-housesystemofrating:Itisbaseduponfourfactors-systemcomprisingskill,effort,conditionsandconsistencyandarrivesatcumulativerating.

    - Syntheticrating:Itistheratioofthestandardtomefortheelementtothatofobservedtime.

    - Objectiverating;Itisdeterminedintwostages-firstthespeedratingandsecondtheadjustmentforjobdifficulties.

    - Psychologicalevaluationofperformancelevel:Inthisratingweconsidertheamountofoxygenconsumed,changeofheartrateetctodetermineratingfactor.Typesofallowancesintimestudy:

- Processallowances:Processallowancestocompensateforenforcedidlenessduetonoworkpowerfailure,faultymaterialortoolsandequipment.

- Personal allowances: It comprises personal needs andfatigue.Fatigueallowancecontains,inturn,aconstantportionandavariableportion.

- Interferenceallowances:Whenaworkerisattendingmorethanonemachine4)Contingency allowances: These allowances are to meet legitimatebutirregularorinfrequentitemsofworkordelays.

5)Specialallowances:Specialallowancesdecidedasapolicymatter,coveringactivitiesthatareessentialforsatisfactoryperformanceofworkbutmaynotbepartofthejob,likestartup,shutdown,changeover,cleaning,setup,toolchangingetc.

## STATISTICALQUALITYCONTROL

Introduction:Qualityisthedeterminingfactorthesuccessofanyproductorservicelargeresourcearecommittedineveryorganizationtoensurequality

Definition:Itisdefinedascustomersatisfactioningeneralandfitnessforuseinparticular.Boththeexternalconsumerwhobuytheproductandservicesandtheinternalconsumersthatis,alldivisionsordepartmentsofthebusinessorganizationareequallyinterestedinthequality.

Statisticalquality control:Theprocessofapplyingstatisticalprinciplestosolvetheproblemofcontrollingthequalitycontrolofaproductorserviceiscalledstatisticalqualitycontrol.

Qualityelements:a)Qualitydesignb)Qualityconformance

- Qualitydesign:Qualityofdesignreferstoproductfeaturesuchasperformance,reliabilitydurability,easeofuse,serviceability

- Qualityconformance:Qualityconformancemeanswhethertheproductmeetsthegivenqualityspecificationornot

Inspection:The process ofmeasuringtheout putandcomparing it tocheck whetheritmeetsthegivenspecifiedrequirementsornot,iscalledinspection.

InspectionMethods:Thefollowingarethemethodsofinspectionbasedonmerits

  - Incominginspection:Inthismethod,thequalityofthegoodsandservicesarrivingintotheorganizationisinspected.Thisensuresthatthematerialsuppliersadheretothegivenspecificationswiththisdefectivematerialcannotenterintotheproductionprocess.Thisfocusesonthevendor’squalityandabilitytosupplyacceptablerawmaterials.

  - Criticalpointinspection:Inspectingatthecriticalpointsofaproductmanufacturegivesvaluableinsightintothecompletelyfunctionalprocess.Atthepointsofmanufacturethatinvolvehighcostsorwhichoffernopossibilityforrepairorrework,inspectioniscrucialfurtheroperationdependontheseresultscriticalpointinspectionhelpstodropthedefectiveproduction,andthereby,facilitateavoidingunnecessaryfurtherexpenditureonthem.

  - Processinspection:Thisisalsocalledpatrollinginspectionorfloorinspectionorrovinginspection.Heretheinspectorgoesaroundthemanufacturingpointsintheshopfloortoinspectthegoodsproducedonrandomsamplebasisfromtimetotime.

  - Fixedinspection:Itprovidesforacentralizedandindependentwhereworkisbroughtforinspectionfrom time totime.Thismethod isfollowedwherethe inspectionequipmentcannotbemovedtothepointsofproductions.

  - Finalinspection:Thisiscentralizedinspectionmakinguseofspecialequipment.Thiscertifiesthequalityofthegoodsbeforetheyareshipped.

ElementsofstatisticalQualityControl:ThetechniqueunderSQCcanbedividedintotwopartsa)Processcontrolb)Acceptancesampling

    - Processcontrol:	Processcontrolisatechniqueofensuringthequalityoftheproductsduringthemanufacturingprocessitself.Ifaprocessconsistentlyproducesitemswithacceptableortolerablerangeofspecification.	Itissaidtobestaticallyundercontrol.	Processcontrolisachievedthroughcontrolcharts.Processcontrolaimstocontrolandmaintainthequalityoftheproductsinthemanufacturingprocess.Statistical controlcharts:	A control chartcompares graphicallythe processperformancedatatocomputedstatisticalcontrollimits.	Thesecontrollimitsactaslimitlinesonthechartcontrolchatsarethetoolstodeterminewhethertheprocessisundercontrolornot.

Thequalityoftheproductionprocessmaybeaffectedbychancecauseorassignablecause.

Chancecause:such causes,which mayormaynotaffectthe manufacturingprocessarecalledchance cause,chance cause cannoteven be identified.Itisnotpossibletoalwaysmaintainthegivenspecification.

AssignableCause:Assignablecausesaffectthequalityoftheproductionprocess.Thesecausescanbeidentifiedandspecified.Causessuchaschangeinthelabourshift,powerfluctuations,orexcessivetoolweararesaidtobeassignablecausesastheyaffectthequalityofmanufacturingprocessindifferentways.

Processcapability:Processcapabilityreferstotheabilitytoachievemeasurableresultsfromacombinationofmachines,tools,methods,materialsandpeopleengagedinproduction.

## Confidencelimitsandcontrollimit:

Confidencelimit:Itindicatetherangeofconfidencelevel.Aconfidencelevelreferstotheprobabilitythatthevalueofmeasurementorparameter,suchaslengthofscrew,iscorrect.

Ex:Ifacomponentisrequiredwithmeasurementof50mm.across,thenthebuyacceptallcomponentsmeasuringbetween48mmand52mmacross,consideringafivepercentconfidencelevel.

Controllimit:Controllimitsarefoundinthecontrolcharts.Therearetwocontrollimits1)Uppercontrollimit(UCL)and2)Lowercontrollimit(LCL).Thesearedeterminedbasedontheprinciplesofnormaldistribution

Ex:Inapilotinvestigationofthelengthofthenailsproducedintheshopfloor,itis

foundthatthemeanlengthXiscm,theS.D3σ,themeasureofvariabilityofthenailsproduced0.2cm.Howdoyouconstructthecontrolchartforthisdata.

+3σUppercontrollimit

4.6

Lengthofnail

4

3.4

x

-3σLowercontrollimit

Samplenumber

Controlchartsfor variables:Avariableisonewhosequalitymeasurementchangesfromunittounit.Thequalityofthesevariablesismeasuredintermsofhardness,thickness,length,andsoon.Thecontrolchartsforvariablesaredrawnusingtheprinciplesofnormaldistribution.Therearetwotypesofcontrolchartsforvariablesx

andRchart.

XandRChart:TheXchartisusedtoshowtheprocessvariationsbasedontheaveragemeasurementofsamplescollected.ItshowsmorelightondiagnosingqualityproblemwhenreadalongwithRchart.Itshowstheerraticorcyclicshiftsinthemanufacturingprocess.Itcanalsofocusonwhentotakearemedialmeasuretoset

rightthequalityproblems.However,collectingdataaboutallthevariablesinvolvesalargeamountoftimeandresources.

TheRchartisbasedontherangeoftheitemsinthegivenample.Ithighlightsthechangesintheprocessvariability.Itisagoodmeasureofspreadorrange.ItshowsbetterresultswhenreadalongwiththeXchart.

Forxcharts:      UCL  = x+A2R	Whenx=Meanof Means

LCL=	x-A2R	R=MeanofsamplerangeA2=Constant

ForRchart:	UCL	=D4R	D4,D3areconstantsLCL	=  D3R

Ristheaverageofsampleranges(Rangesisthedifferencebetweenthemaximumvariableandminimumvariable)

EX:ConstructxandRchartsfromthefollowinginformationandstatewhethertheprocessisincontrolforeachofthefollowingxhasbeencomputedfromasampleof5unitsdrawnatanintervalofhalfanhourfromanongoingmanufacturingprocess.

| Samples | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| x | 24 | 34 | 35 | 39 | 26 | 29 | 13 | 34 | 37 | 29 |
| R | 23 | 39 | 14 | 5 | 20 | 17 | 21 | 11 | 40 | 10 |

Solution:Themeanofmeans	x = x

# n

300

=

# 10

=30

Riscalculatedas	R  =

R

# n

20020

10

xChart: xhcartUCLandLCL compute atsamplesize5A 2tablevalueis0.58

UCL=x+A2R=30+(0.58x20)=41.6LCL	=D3R =30– (0.58x20)=18.4

60

50

40

Measurement

ofvariables	30

20

10

UCL=41.6

x=30

LCL=18.4

0

1	2	3	4	5	6	7	8	9	10

Numberofsamples

RChart: RchartUCLandLCLcomputeatsamplesize5,D4tablevalueis2.11andD3tablevalueis0

UCL	=D4R =2.11 x20=42.2LCL	=  D3R =0 x20=0

45

25

20

Measurement

ofvariables	15

UCL=42.6

R=42.6

10

5

0

1	2	3	4	5	6	7	8	9	10

Numberofsamples

LCL=0

Therefore3,7pointstheprocessisoutofcontrol.

Controlchartsforattributes:Thequalityofattributescanbedeterminedonthebasisof‘Yes’or‘No’,‘Go’or‘Nogo’.Inotherwords,incaseofamirrorglass,evenifthereisonescratchitisnotconsideredtobeaqualitymirror,insuchacasequalityisdecidedbaseonwhetherthemirrorhasanyscratchornot.

Thecontrolchartsforattributesare‘C’chartand‘P’charts

‘C’Chart:‘C’chartisusewherethereanumberdefectsperunit.Thiscontrolchartscontrolsthenumberofdefectsperunit.Herethesamplesizeshouldbeconstant. Thiscalculateasbelow.

c

UCL =c+3

Wherethec=

c

andLCL=c-3

Totalnumberof defectsinallthesamples

Totalnumberof

samplesinspected

## Ex:

| SampleNumber | No.ofdefects | SampleNumber | No.ofdefects |
| --- | --- | --- | --- |
| 1 | 5 | 11 | 4 |
| 2 | 4 | 12 | 6 |
| 3 | 9 | 13 | 7 |
| 4 | 7 | 14 | 3 |
| 5 | 8 | 15 | 5 |
| 6 | 9 | 16 | 3 |
| 7 | 4 | 17 | 3 |
| 8 | 5 | 18 | 1 |
| 9 | 2 | 19 | 7 |
| 10 | 6 | 20 | 2 |

Totalnumberofdefects	=	100

c=1005

c

5

20

UCL =c+3

= 5 +3

=11.69

c

LCL=c-3

= 5 -3	=0

5

LCL=0means,LCLgotnegativevalue,takeitasequaltozero

20

15

10

No.defects

per each	8

sampledunit

6

5

4

2

0

2	4	6

8	10

12 14	15	18

UCL=11.69

c=5

LCL=0

20

Numberofsamples

‘P’Chart:‘P’Chartisusedwherethereisdateaboutthenumberofdefectivespersample.Itisalsocalledfractiondefectivechartorpercentagedefectiveschart.Hereeachitemisclassifiedon‘goornogo’basisthatisgoodorbad.Henceifthesamplesizeislarger,theresultscouldbebetter.

UCL=LCL=

Whereaveragedefective(p)=

Totalno.of

defective

found

Totalno.of

pieces inspected

‘n’=Numberofpiecesinspectedperday

Ex:Foreachofthe14daysanumberofmagnetsusedinelectricrelaysareinspectedandthenumberofdefectivesisrecorded.Thetotalnumber ofmagnetstestedis14,000.Thefollowingaretheparticularofthenumberofdefectivesfoundeveryday.

| Daynumber | Numberofdefective | Daynumber | Numberofdefective |
| --- | --- | --- | --- |
| 1 | 100 | 8 | 120 |
| 2 | 50 | 9 | 60 |
| 3 | 150 | 10 | 140 |
| 4 | 200 | 11 | 50 |
| 5 | 150 | 12 | 70 |
| 6 | 50 | 13 | 40 |
| 7 | 80 | 14 | 40 |

## Solution:

Totalnumberofdefectives=14000

Theaveragesamplesize(n)perday=14000/14days=1000

Percentageofdefectiveperday=

Totalno.ofdefectivefoundperday

Totalno.of

pieces inspected

perday

| Daynumber | Percentageofdefectives | Numberofdefective | Percentageofdefectives | Daynumber | Numberofdefective |
| --- | --- | --- | --- | --- | --- |
| 1 | 100/1000=0.10 | 100 | 120/1000=0.12 | 8 | 120 |
| 2 | 50/1000=0.05 | 50 | 60/1000=0.06 | 9 | 60 |
| 3 | 150/1000=0.15 | 150 | 140/1000=0.14 | 10 | 140 |
| 4 | 200/1000=0.20 | 200 | 50/1000=0.05 | 11 | 50 |
| 5 | 150/1000=0.15 | 150 | 70/1000=0.07 | 12 | 70 |
| 6 | 50/1000=0.05 | 50 | 40/1000=0.04 | 13 | 40 |
| 7 | 80/1000=0.08 | 80 | 140/1000=0.14 | 14 | 40 |

(p)=

Totalno.ofdefectivefound

Totalno.of

pieces inspected

0.110.1

1000

1400/14000=0.1

UCL=0.1+3

=0.4

0.110.1

1000

LCL=0.1 -3	=0

Percentageofdefective

0.45

0.40

0.30

0.20

0.15

0.10

0.05

UCL=0.4

p0.1

LCL=0

1	5	10	15

DayNumber

AcceptanceSampling:Acceptancesamplingisatechniqueofdecidingwhethertoacceptthewholelotornotbasedonthenumberofdefectivesfromarandomdrawnsample.

Itiswidelyuseinbuyingfoodproducts,suchasrice,wheatetc.Beforebuyingtherandomsamplesdrawnfromthebagsofsayricearetested.Ifthequalityofsampledrawnlooksgoodorfreefromdefectsthenaccordingtotherequirementtheentirebagorpartofitcanbebrought

Theprocessofacceptancesamplingthroughoperatingcharacteristiccurve(OCC)

Operatingcharacteristiccurve(OCC):Thegraphicalrelationshipbetweenpercentagedefectiveinthelotsbeingsubmittedforinspectionandtheprobabilityacceptanceistermedas“operatingcharacteristicofaparticularsamplingplan”

Probabilityofacceptancepercentage

100

80

60

40

20

C=1(Acceptancenumber)n=50(samplesize)

1	2	3	4	5 6	78910

Actualpercentageofdefectives

Itgivesaclearpictureabouttheprobabilityofacceptanceoflotforvariousvaluesofpercentdefectivesinthelot.Theprobabilityofacceptanceofalotishighforlowvaluesofactualpercentagedecreaseanditislowforhighvaluesofactualpercentagedefectives.

ConstructionofOCcurve:Todevelopasamplingplanforacceptancesampling,anappropriateO.CcurvemustbeselectedtoconstructanOCcurveanagreementhastobereachedbetweentheproducerandtheconsumeronthefollowingfourpoint.

      - Acceptablequalitylevel(AQL):Thisisthemaximumproportionofdefectivesthatwillmakethelotdefinitelyacceptable.

      - Lottolerancepercentagedefective(LTPD):Thisisthemaximumproportionofdefectivesthatwillmakethelotdefinitelyunacceptable.

      - Producersrisk(α):Thisistherisk,theproduceriswillingtotakethatlotsofthequalitylevelAQLwillberejected,eventhough,theyareacceptableusuallyα=5%

      - Consumerrisk(β):Thisistherisk,theconsumeriswillingtotakethatlotsofthequalitylevelLTPDwillbeaccepted,eventthough,theyareactuallyunacceptableusuallyβ=10%.

Probabilityofacceptance

Producersriskα=10%

α	AQL=20%

LTPD=60%

Consumersriskβ=10%

β

20%	60%	100%

Percentageofdefectives(p)

Samplingplans:Basedonthenumberofsamplesdrawnfortakingaccept/rejectdecisions,thesamplingmethodsareused.Therearefourmethodsofacceptancesamplings.

- Singlesamplingplan:Alotisacceptedorrejectedonthebasisofasinglesampledrawnfromthatcost

- Doublesamplingplan:Ifitisnotpossibletodecidethefateofthelotonthebasisoffirstsample,asecondsampleisdrawnandthedecisionistakenonthebasisofthecombinedresultsoffirstandsecondsample.

- Multiple samplingplan:Alotis accepted orrejectedbasedupontheresultobtainedfromseveralsamples(ofparts)drawnfromthelot.

- Sequentialsamplingplan:(Itembyitemanalysis)

Sequentialsamplinginvolvesincreasingthesamplesizebyonepartatatimetillthesamplebecomeslargeenoughandcontainssufficientnumberofdefectivestodecideintelligentlywhethertoacceptorrejectthelot.