ACKNOWLEDGEMENT

This Project work has been an experience that shall stand worthy and valuable for us in years to come, we wish to express our deep sense of gratitude towards   JETKING COMPUTER  HARDWARE NETWORKING INSTITUTE.

for giving us an opportunity to undertake the project.

We would like to acknowledge all those who shall go through this Project that, this would have never been a success without the help of those who always stood beside us through the project.

We take the pleasure in expressing our deep sense of gratitude towards our project guide XX who always boosted our moral whenever we approached his for guidance, who without putting us in too many ups & downs or hassles, always lend a hand of support in the right direction and for her thoughtful guidance and inspiration for smooth mobilization and completion of this project.

Index

- Introduction

- Components Used

- Procedure

- Diagram

- Conclusion

- Acknowledgement

7.      Bibliography

CLAP SWITCH THEORY

Here is a circuit for electronics hobbyists that can switch on & off a light, Fan, Radio etc. by the sound of clap.The sound of clap is received by a small microphone that is shown biased by resistor R1 in the circuit. The microphone changes sound wave in to electrical wave which is further amplified by Q1. Transistor Q1 is used as common emitter circuit to amplify weak signals received by the microphone. Amplified output from the collector of transistor Q1 is then feed to the Bistable Multivibrator circuit also known as flip-flop.

Flip flop circuit is made by using 2 Transistor, in our circuit Q2&Q3. In a flip-flop circuit, at a time only one transistor conduct and other cut off and when it gets a trigger pulse from outside source then first transistor is cutoff and 2nd transistor conducts. Thus output of transistor is either logic-0 or logic-1 and it remains in one state 0 or 1 until it gets trigger pulse from outer source.

The pulse of clap which is trigger for flip-flop makes changes to the output which is complementary (reverse). Decision of flip-flop which is in the low current form is unable to drive relay directly so we have used a current amplifier circuit by using Q4 which is a common emitter circuit. Output of Q4 is connected to a Relay (Electromagnetic switch), works like a mechanical switch. With the help of a relay it is easy for connecting other electrical appliance.

The relay contact is connected to the power line and hence turns on/off any electrical appliance connected all the way through relay.

.

CONCLUSION

From the study of CLAP SWITCH , we find that the clap switch system is most important and useful tp us .It is used to turn ON or OFF any electrical circuit by a simple clap eliminating the need to turn the switch ON or OFF manually.

INTRODUCTION

Here is a circuit for electronic hobbyists that can switch ON and OFF a light , a fan or a Radio etc…

The sound of clap is received by a small microphone that is shown biased by resistor R1  in the circuit.  The signal is further amplified by transistor Q1 ,Q2 ,Q3 . The relay contact is connected to the power line and hence turn ON and OFF any electrical device connected at the output socket .

PARTS LIST :-

Resistors:			Capacitors:			Misc:

R1		15 KΩ	C1	=1000µf/16v		T1	=12v/300mA	Transformer

R2, R11, R12   2.2MΩ	C2	=.01µf					Mic	=Condenser Microphone

R3		270 KΩ	C3, C4 =.047µf		K1	=12V Relay

R4		3.3 KΩ		Semiconductors:           B1	=Bulb or Load

R5, R6		1.5 KΩ		Q1, Q2, Q3	= BC548

R7, R8		10 KΩ	Q4		= BC368

R9, R10	27 KΩ		D1, D5		= IN 4007

R13		2.2 KΩ		D2, D3, D4	= IN 4148

BIBLIOGRAPHY

1) References of sites www.elecrtonic.com

2) Basic electronic book

3)Electronic media magazines

4) Other institutional book from library,

CIRCUIT DIAGRAM

For power supply , we have made 12Volt eliminator with the help of Transformer T1, Diode D1 and capacitor C1.It is a half wave rectifier.