“Wireless equipment control using AT89C51”

ABSTRACT

Here is a microcontroller based wireless equipment controller that can switch on or off upto four devices at a desired time interval set by the user in the transmitter. The devices can be controlled remotely from a distance  upto 30 metres from the transmitter. In the transmitter, an LCD module is used to show the device numbers and preset control time for the devices (00 to 99 seconds). The 8-bit AT89C51 microcontroller is the main controlling part of the transmitter section. It is connected to the LCD module, input switches and encoder IC (HT12E). The device control program is stored in the memory of the microcontroller to control the devices as per the time out settings done through input switches S1 through S4. The TRX -434 RF transmitter module uses a digital modulation technique called ASK (Amplitude Shift Keying) or on-off keying.

The RX-434 radio receiver module receives the ASK signal from TRX-434. The HT12D decoder demodulates the received address and data bits.

Concepts of wireless RF communication and automation with AT89C51 microcontroller are used here.

CONTENTS

Chapter 1 Introduction…………………………………………………….........................1

1.1 Transmitter Section...…………………………..............................................1

1.2 Receiver Section…….………………………………….……………………3

1.3 Applications………………………………………………………………….4

Chapter 2 Hardware Description..………………………………………………………..,5

2.1 Circuit Description……………………………..………………………….…5

2.1.1 Transmitter Circuit…………………………………………………………5

2.1.2 Receiver Circuit…………………………………………………………….7

2.2 Circuit Operation………………………………………………………….…..8

Chapter 3 Software Programming……….………………….……………........................10

3.1 Software……………………………………………………………….……10

3.2Programming the Flash……………………………………………………..13

3.2.1 Ready/Busy…………………………………………………………….....14

3.2.2 Program Verify……………………………………………………….…....14

3.2.3 Chip Erase…………………………………………………….………...…15

3.2.4 Reading Signature Bytes…………………………………………..…..….15

3.2.5 Power Down Mode………………………………………………….……..16

3.2..6 Program Memory Lock Bits……………………………………………....16

Chapter 4 PCB layout……………………………………………………..........................17

4.1 Preparing Circuit Layout……………………………………………………17

4.2 Flowchart……………………………………………………………………19

Chapter 6 Conclusion & Future Scope ……..……………………….……………………24

References

Appendix

FIGURE INDEX

Figure 1.1 Block diagram of Transmitter Circuit…………………………………….1

Figure 1.2.1  ASK concept for the RF transmitter module….……………..…………2

Figure1.2 Block diagram of receiver section …………………………………………3

Figure 2.1.1 Trasmitter circuit………….…………………..…………………………5

Figure 2.1.2 Receiver circuit………………………………………………………….7

Figure 3.1 Types of programming language………………………………………….10

Figure 3.2 Flowchart of software program………………………………….……......11

Figure 3.2.1 Programming Flash……………………………………………….…….13

Figure 3.2.2 Verify Flash………………...……………………..…...……………..…14

Figure 4.2 flow chart for preparation of pcb layout………………………………….19

Figure 5.1 PCB Layout of transmitter………………………………………………..20

Figure 5.2 Component Layout transmitter .…………………………………….........21

Figure 5.3 PCB Layout of receiver……………………………………………………22

Figure 5.4 Component Layout receiver………………………………………………..23

CHAPTER 1

INTRODUCTION

The system comprises a transmitter and a receiver as described belows

1.1 TRANSMITTER SECTION :

Fig.1.1  shows the block diagram of the transmitter section. Four push button switches (S1 through S4) are used as input to select the devices and sets the time out in the transmitter section. These are designated as up, down, enter and run keys respectively. The time out data is transferred over the RF wireless link to the receiver section.

Fig. 1.1 –Block Diagram Of Transmitter Section For Wireless Equipment Control

The 8-bit AT89C51 microcontroller is the main controlling part of the transmitter section. It is connected to the LCD module, input switches and encoder IC (HT12E). The device control program is stored in the memory of the microcontroller to control the devices as per the time out settings done through input switches S1 through S4.

A two-line, 16 character LCD module shows the status of the main program that is running inside the microcontroller. The HT12E is an 18 pin DIP  package encoder IC that encodes 4-bit data and sends it to the TRX -434 RF transmitter module.

The TRX -434 RF transmitter module uses a digital modulation technique called ASK (Amplitude Shift Keying) or on-off keying. In this technique, whenever logic ‘1’ is to be sent, it is modulated with carrier signal (434 MHz). This modulated signal is then transmitted through the antenna. The waveforms in fig. 2 depict the ASK concept.

Fig. 1.1.2 – ASK Concept For The RF Transmitter Module

1.2 RECEIVER SECTION :

Fig. 3 shows the block diagram of receiver section. The 12V DC supply, used along with the 5V regulator, can be provided by a 12V battery or power adaptor.

The RX-434 radio receiver module receives the ASK signal from TRX-434. The HT12D decoder demodulates the received address and data bits. IC CD4519 is a quadruple two-input  multiplexer that selects appropriate data bits to control the devices. The ULN 2003 relay driver consists of seven npn darlington pairs that feature high- voltage outputs with common cathode clamps diodes for switching the inductive loads. The collector –current  rating of a single darlington pair is 500 mA.

Fig. 1.2 –Block Diagram Of Receiver Section For Wireless Equipment Control

1.3 APPLICATIONS :

- Used for controlling home appliances.

- Used for controlling industrial instrumentation.

- Used for controlling devices in labs.

CHAPTER 2

HARDWARE DISCRIPTION

2.1 CIRCUIT DISCRIPTION :

2.1.1 TRANSMITTER CIRCUIT: –

Fig. 4 shows the transmitter circuit. The microcontroller reads the input data from the switches S1 through S4. At its ports-2 pins 21 through 24 and displays it on the LCD. Port 3 provides  read data to the encoder IC HT12E  at pins 10 through 13. The microcontroller is programmed to control the input and output data.

Fig. 2.1.1 –Transmitter Circuit

When the push button switches (S1 through S4) are open, logic ‘0’ is constantly fed to the respective port pins to the microcontroller. When any of the buttons is pressed, logic ‘1’ is fed to the respective port pin of the microcontroller,

The device control program stored in the memory of the microcontroller activates and executes as per the functions defined in the program for respective input switches. Data input AD8 through AD11 (pins 10 through 13) HT12E are connected to the microcontroller. Pins 1 through 8 (A0 through A7) of the IC are address inputs. Shorting of address pins using switches to either Vcc or Gnd enables different address selections of for data transmission. Here we have connect them to 5V. Since address pins are connected to 5V, the address is set to 255d (in decimal). If you were to connect all the address pins to ground the address would be 00d. Thus there are 256 possible addresses available. So you can set up switches to control one or more of the encoder address  pins.

Pin 14 is a transmit –enable (TE) input pin. The encoder will send data only when pin 14 is connected to the ground. Whenever button is pressed, logic ‘0’ is sent to this pin through the microcontroller, thus activating it and enabling transmission.

Pin 17 is the data out (D out) pin that sends the serial stream of pulses containing the address and data it is connected to the data input pin of the TRX RF module. The time out control is set using in-put keys S1 through S4 to turn on/off the devices at predetermined time. The default time for all the devices is  ‘00’ seconds. So using ‘up’ key you can increment time by one second and using ‘down’ key you can decrement time by one second down. At the same time LCD module shows the current status of increments and decrements.

When the time out for a device is set, press ‘ent’ key so that the program control transfers to the next device for time out settings. In the same way the three remaining time out settings must be  done before pressing ‘run’ key. When ‘run’ key is pressed it executes the device control program sub routine in the microcontroller and the program automatically collects the time out information collected by the user and sends the processed data to encoder IC HT12E.  The encoder IC sends the data to (D in) of the RF transmitter module. The data is transmitted by the TRX- 434 module to receiver section through antenna.

2.1.2 RECEIVER CIRCUIT

Fig. 5 shows the receiver circuit. The RF receiver circuit ( RX- 434) module can receive the signal transmitted by the transmitter from a distance of upto 9 metres ( 30 feet). The range can be increased up to 30 metres using a good antenna. D out pin of RX-434 module is connected to the D in pin of decoder IC (HT12D). D in pin receives the address and data bits serially from the RF module. Decoder separates data and address from the received information. It accepts data only if the received address matches with the assigned to the encoder address (HT12E). The HT12D decoder receives serial addresses and data from the encoder that are transmitted by a carrier signal over the RF medium. The decoder compares the serial input data three times continuously with its local address. If no error or unmatched codes are found, the input data codes are decoded and transferred to the output pins. The HT12D provides four latch type data pins whose data remains unchanged until new data is received. Data pins D8 through D11 of the decoder sends 4-bit data to CD4519 multiplexer IC.

Fig. 2.1.2 –Receiver Circuit

This IC CD4519 multiplexer provides four multiplexing circuits with common select inputs (S a and S b) ; each contains two inputs (A n , B n) and one output ( O n). It may be used to selects 4-bit information from one of the two sources. There are 8 input lines ( A0 through A3 and B0 through B3) , of which four (A0 through A3) are permanently connected to Vcc through resistor R19, while the rest four ( B0 through B3 ) are connected to data output lines of the decoder.

The select inputs can be connected to either Vcc or VT  pin  (pin 17) for latch or momentary operation. Jumper switch (JS) is used to select between latch or momentary operation. When latch mode is selected, data present at the output pins is latched. When the momentary mode is selected, the data presented at output pins is available as long as VT pin remains active high. As soon as VT pin becomes active low, the respective relay de-energies.

The latched output data from the multiplexer is fed to the relay driver IC ULN2003, to control up to four devices through relays ( RL1 through RL4). VT pin is connected to LED4 through IC6 to indicate the status of VT signal when it is active high.

2.2) CIRCUIT  OPERATION

When the system is switched ON, the startup message “press any key” appears on the LCD screen. When any key is pressed by the user, the LCD displays the message “ to set time out press ent ”.  Pressing  “ent” key displays the following  messages on the LCD with  a cursor blinking near the first device ‘ D1 T ’ :

D1_T =                                         D2_T =

D3_T =                                         D4_T =

Use ‘up’ and ‘down’ to set the time for controlling the devices. The set time for each device on the LCD screen looks like this :

D1_T = 10                                   D2_T = 20

D3_T = 30                                   D4_T = 40

Now press ‘ent’ key followed by the ‘run’ key. A device control sub routine executes and sends the data to the RF module, which transmits the data through ANT system. You can set  maximum of 99 seconds as the control time for the device. If you set it to 00 , a particular device is turned ON for infinite time.

CHAPTER 3

SOFTWARE PROGRAMMING

3.1 SOFTWARE

- There are other languages also but these two are widely used.

- Now a days even assembly language is not preferred but Embedded C Is being used extensively for development in industries.

- The compilers of these languages convert the codes into HEX file  which is burnt into the microcontroller using a Burner.

- It is a high level language.

- The compiler used for Embedded C is Keil C51.

- It is easy to understand and work with it.

- The compiler is used to covert High Level Language into Machine  Language.

Fig. 3.1 Types Of Programming Language

The software flowchart programmed in the microcontroller of the transmitter section is shown in fig. It is written in Assembly language and compiled using ASM51 software to generate the hex code. The hex program can be burnt into the AT89C51 microcontroller by using any standard program available in the market.

The software program is designed to accept the input from the user as well as control the devices. It identifies the key pressed and displays the key code on the LCD module. In the program, the LCD module is initialized first. As soon as the time-out is set, all the four devices turn off at preset time.

In this project, the time-out range is 00 to 99 seconds, which can be easily modified to extend the time duration in the time delay subroutine of Assembly code. Port 0 is configured as output port and interfaced with the RF module through encoder IC1. Port 1 is used for LCD interface and port 2 is used for the input from push-to-on switches.

Fig.3.2:-Flowchart Of Software Program

3.2 PROGRAMMING THE FLASH

The AT89C51 is normally shipped with the on-chip Flash memory array in the erased state (that is, contents = FFH) and ready to be programmed. The programming interface accepts either a high-voltage (12-volt) or a low-voltage (VCC) program enable signal. The low voltage programming mode provides a convenient way to program the AT89C51 inside the user’s system, while the high-voltage programming mode is compatible with conventional third party Flash or EPROM programmers. The AT89C51 is shipped with either the high-voltage or low-voltage programming mode enabled. The AT89C51 code memory array is programmed byte by byte in either programming mode. To program any non-blank byte in the on-chip Flash Memory, the entire memory must be erased using the Chip Erase Mode.

Programming Algorithm: Before programming the AT89C51, the address, data and control signals should be set up according to the Flash programming mode table and Figures . To program the AT89C51, take the following steps:

1. Input the desired memory location on the address lines.

2. Input the appropriate data byte on the data lines.

3. Activate the correct combination of control signals.

4. Raise EA/VPP to 12 V for the high-voltage programming mode.

5. Pulse ALE/PROG once to program a byte in the Flash array or the lock    bits. The byte-write cycle is self-timed and typically takes no more than 1.5 ms. Repeat steps 1 through 5, changing the address and data for the entire array or until the end of the object file is reached. Data Polling: The AT89C51 features Data Polling to indicate the end of a write cycle. During a write cycle, an attempted read of the last byte written will result in the complement of the written datum on PO.7. Once the write cycle has been completed, true data are valid on all outputs, and the next cycle may begin. Data Polling may begin any time after a write cycle has been initiated.

Fig 3.2.1 Programming Flash

3.1 Ready/Busy:

The progress of byte programming can also be monitored by the RDY/BSY output signal. P3.4 is  pulled low after ALE goes high during programming to indicate BUSY. P3.4 is pulled high again when programming is done to indicate READY.

3.2 Program Verify:

If lock bits LB1 and LB2 have not been programmed, the programmed code data can be read back via the address and data lines for verification. The lock bits cannot be verified directly. Verification of the lock bits is achieved by observing that their features are enabled.

Fig 3.2.2 Verify Flash

3.3 Chip Erase:

The entire Flash array is erased electrically by using the proper combination of control signals and by holding ALE/PROG low for 10 ms. The code array is written with all “1"s. The chip erase operation must be executed before the code memory can be re-programmed.

3.4 Reading The Signature Bytes:

The signature bytes are read by the same procedure as a normal verification of locations 030H, 031H, and 032H, except that P3.6 and P3.7 must be pulled to a logic low. The values returned are as follows:

(030H) = 1EH indicates manufactured by Atmel

(031H) = 51H indicates 89C51

(032H) = FFH indicates 12 V programming

(032H) = 05H indicates 5 V programming.

3.5 Power Down Mode

In the power down mode the oscillator is stopped, and the instruction that invokes power down is the last instruction executed. The on-chip RAM and Special Function Registers retain their values until the power down mode is terminated. The only exit from power down is a hardware reset. Reset redefines the SFRs but does not change the onchip RAM. The reset should not be activated before VCC is restored to its normal operating level and must be held active long enough to allow the oscillator to restart and stabilize.

3.6 Program Memory Lock Bits

On the chip are three lock bits which can be left unprogrammed (U) or can be programmed (P) to obtain the additional features listed in the table .When lock bit 1 is programmed, the logic level at the EA pin is sampled and latched during reset. If the device is powered up without a reset, the latch initializes to a random value, and holds that value until reset is activated. It is necessary that the latched value of EA be in agreement with the current logic level at that pin in order for the device to function properly.

CHAPTER 4

PCB LAYOUT

The entire  circuit  can  be  easily  assembled  on  a  general  purpose  P.C.B.  board respectively.  Layout of desired diagram and preparation is first and most important operation in any printed  circuit  board manufacturing process. First of all layout of component side is to be made in accordance with available components dimensions. The following points are to be observed while forming the layout of P.C.B.

1.  Between two components, sufficient space should be maintained.

2.  High voltage/max dissipated components should be mounted at sufficient distance from semiconductor and electrolytic capacitors.

3.  The most important points are that the components layout is making proper compromise with copper side circuit layout. Printed circuit board (P.C.B.s) is used to avoid most of all the disadvantages of conventional  breadboard. These also avoid the use of thin wires for connecting the components; they are small in size and efficient in performance.

An actual-size,single-side PCB layout of the transmitter for wireless equipment control using microcontroller is shown in fig.and its component layout in fig.The actual-size,single-sided PCB layout for receiver circuit is shown in fig.and its component layout in fig.

4.1 PREPARING CIRCUIT LAYOUT

First of all the actual size circuit layout is to be drawn on the copper side of the copper clad board. Then enamel paint is applied on the tracks of connection with the help of a shade  brush.  We  have  to  apply  the  paints  surrounding  the  point  at  which  the connection  is  to  be  made.  It  avoids  the  disconnection  between  the  leg  of  the component and circuit track. After completion of painting work, it is allowed to dry.

DRILLING After completion of painting work, holes 1/23inch(1mm) diameter are  drilled at desired points where we have to fix the components.

ETCHING

The removal of excess of copper on the plate apart from the printed circuit is known as etching. From this process the copper clad board wit printed circuit is placed in the solution of FeCl with 3-4 drops of HCL in it and is kept so for about 10 to 15 minutes and is taken out when all the excess copper is removed from the P.C.B.

SOLDERING

Soldering is the process of joining two metallic conductor the joint where two metal conductors are to be join or fused is heated with a device called soldering iron and then as allow of tin and lead called solder is applied which melts and converse the joint. The solder cools and solidifies quickly to ensure is good and durable connection between the jointed metal converting the joint solder also present oxidation.

4.2 FLOWCHART

Single sided copper clad copper

Manually cut the board into required size

Clean the Board

Develop the layout of the circuit

Etch unwanted copper

Drill holes

Solder the components on the boed BObBbbbbbbobboed PCB

Fig:4.2 Flow Chart For Preparation Of Pcb Layout

Fig:-4.1. PCB Layout For Transmitter

Fig:-4.2. Component Layout For Transmitter

Fig:-4.3. PCB Layout For Receiver

Fig:-4.4. Component Layout for receiver

CHAPTER 5

CONCLUSION & FUTURE SCOPE

The system is small, simple and good for wireless equipment control. The microcontroller based equipment controller can switch on or off up to four devices at desired time interval set by user in the transmitter. The devices can be controlled remotely from the distance up to 30 metres from the transmitter. The RF receiver module can receive the signal transmitted from a distance up to 9 metres(30feet). The range can be increased up to 30 metres using a good antenna. In this project the time out range is 00 to 99 seconds, which can easily be modified to extend the time duration in the delay subroutine of the assembly language code.

FUTURE SCOPE

The electrical devices can be controlled using wireless equipment control without the use of wires. The messiness caused by the wires is reduced. This is cost-effective also. The device can switch on or off up to four devices at a desired time interval set by the user in the transmitter. The number of devices can be increased by increasing the relay. The devices can be controlled remotely from a distance  up to 30 metres from the transmitter. For increasing the range a good antenna with longer range can be used. The source code can also be written in embedded C language which makes error detecting in the code easier.

REFERENCES

- microcontroller51.blogspot.com

- www.articlesnatch.com

- www.electronicsforu.com/efycodes/efy-codes.zip

- www.alldatasheets.com

- www.google.co.in

- www.datasheetcatalogue.com

- www.efymag.com

- www.wikipedia.com

APPENDIX

SOURCE CODE

;"Wireless Automation Control System"

;Main Program

$MOD51

DB0	EQU	P1.0

DB1	EQU	P1.1

DB2	EQU	P1.2

DB3     EQU	P1.3

DB4	EQU	P1.4

DB5	EQU	P1.5

DB6	EQU	P1.6

DB7	EQU	P1.7

EN	EQU	P3.4

RW	EQU	P3.3

RS	EQU	P3.2

UP	EQU	P2.0

DOWN	EQU	P2.1

ENT	EQU	P2.2

RUN	EQU	P2.3

DATA1	EQU	P1

PACK1	EQU	60H

PACK2	EQU	61H

PACK3	EQU	62H

PACK4	EQU	63H

RAM1	EQU	70H

RAM2	EQU	71H

RAM3	EQU	72H

RAM4	EQU	73H

RAM5	EQU	74H

RAM6	EQU	75H

RAM7	EQU	76H

RAM8	EQU	77H

ORG	00H

LJMP 	MAIN

ORG 	150H

MAIN:		SETB	P0.4

MOV	P0,#1FH

MOV	P0,#0FH

SETB	P0.4

MOV	P0,#10H

MOV	P0,#00H

SETB	P0.4

LCALL	INIT_LCD	;LCD Initialization

MOV	A,#80H

LCALL	CMD

MOV	A,#'P'

LCALL	WRITE_TEXT

MOV	A,#'R'

LCALL	WRITE_TEXT

MOV	A,#'E'

LCALL	WRITE_TEXT

MOV	A,#'S'

LCALL	WRITE_TEXT

MOV	A,#'S'

LCALL	WRITE_TEXT

MOV	A,#' '

LCALL	WRITE_TEXT

MOV	A,#'A'

LCALL	WRITE_TEXT

MOV	A,#'N'

LCALL	WRITE_TEXT

MOV	A,#'Y'

LCALL	WRITE_TEXT

MOV	A,#' '

LCALL	WRITE_TEXT

MOV	A,#'K'

LCALL	WRITE_TEXT

MOV	A,#'E'

LCALL	WRITE_TEXT

MOV	A,#'Y'

LCALL	WRITE_TEXT

MOV	A,#'!'

LCALL	WRITE_TEXT

MOV	A,#'!'

LCALL	WRITE_TEXT

MOV	A,#'!'

LCALL	WRITE_TEXT

SETB	P0.4

MOV 	P2,#0FFH	;P2 AS INPUT

KEEP:		MOV	A,P2		;READ	P2

ANL	A,#0FH

JZ	KEEP

LCALL	DELAY		;Debounce check

MOV	A,P2

ANL	A,#0FH

JZ	KEEP

LCALL	CLEAR_LCD

MOV	A,#80H		;set cursor to frist position.

LCALL	CMD

MOV	A,#'T'

LCALL	WRITE_TEXT

MOV	A,#'O'

LCALL	WRITE_TEXT

MOV	A,#' '

LCALL	WRITE_TEXT

MOV	A,#'S'

LCALL	WRITE_TEXT

MOV	A,#'E'

LCALL	WRITE_TEXT

MOV	A,#'T'

LCALL	WRITE_TEXT

MOV	A,#' '

LCALL	WRITE_TEXT

MOV	A,#'T'

LCALL	WRITE_TEXT

MOV	A,#'I'

LCALL	WRITE_TEXT

MOV	A,#'M'

LCALL	WRITE_TEXT

MOV	A,#'E'

LCALL	WRITE_TEXT

MOV	A,#' '

LCALL	WRITE_TEXT

MOV	A,#'O'

LCALL	WRITE_TEXT

MOV	A,#'U'

LCALL	WRITE_TEXT

MOV	A,#'T'

LCALL	WRITE_TEXT

NOP

NOP

NOP

NOP

NOP

NOP

NOP

MOV	A,#0C0H

LCALL	CMD

LCALL	WAIT_LCD

MOV	A,#'P'

LCALL	WRITE_TEXT

MOV	A,#'R'

LCALL	WRITE_TEXT

MOV	A,#'E'

LCALL	WRITE_TEXT

MOV	A,#'S'

LCALL	WRITE_TEXT

MOV	A,#'S'

LCALL	WRITE_TEXT

MOV	A,#' '

LCALL	WRITE_TEXT

MOV	A,#'!'

LCALL	WRITE_TEXT

MOV	A,#'E'

LCALL	WRITE_TEXT

MOV	A,#'N'

LCALL	WRITE_TEXT

MOV	A,#'T'

LCALL	WRITE_TEXT

MOV	A,#'!'

LCALL	WRITE_TEXT

H1:		JB	ENT,G1

SJMP	H1

G1:		LCALL	DELAY

JB	ENT,T_SET_D

SJMP	H1

T_SET_D:	LCALL	CLEAR_LCD

MOV	A,#80H		;set cursor to frist position.

LCALL	CMD

MOV	A,#'D'

LCALL	WRITE_TEXT

MOV	A,#'1'

LCALL	WRITE_TEXT

MOV	A,#'_'

LCALL	WRITE_TEXT

MOV	A,#'T'

LCALL	WRITE_TEXT

MOV	A,#'='

LCALL	WRITE_TEXT

MOV	A,#88H

LCALL	CMD

MOV	A,#'D'

LCALL	WRITE_TEXT

MOV	A,#'2'

LCALL	WRITE_TEXT

MOV	A,#'_'

LCALL	WRITE_TEXT

MOV	A,#'T'

LCALL	WRITE_TEXT

MOV	A,#'='

LCALL	WRITE_TEXT

MOV	A,#0C0H

LCALL	CMD

MOV	A,#'D'

LCALL	WRITE_TEXT

MOV	A,#'3'

LCALL	WRITE_TEXT

MOV	A,#'_'

LCALL	WRITE_TEXT

MOV	A,#'T'

LCALL	WRITE_TEXT

MOV	A,#'='

LCALL	WRITE_TEXT

MOV	A,#0C8H

LCALL	CMD

MOV	A,#'D'

LCALL	WRITE_TEXT

MOV	A,#'4'

LCALL	WRITE_TEXT

MOV	A,#'_'

LCALL	WRITE_TEXT

MOV	A,#'T'

LCALL	WRITE_TEXT

MOV	A,#'='

LCALL	WRITE_TEXT

H2:		JB	ENT,G2

SJMP	H2

G2:		LCALL	DELAY

JB	ENT,STIME

SJMP	H2

STIME:		MOV	A,#85H		;SET CURSOR TO L=1,P=5

LCALL	CMD

LCALL	UP_DOWN

MOV	RAM1,A		;D_1

MOV	A,#86H

LCALL	CMD

LCALL	WAIT_LCD

LCALL	UP_DOWN

MOV	RAM2,A		;D_1

MOV	A,#8DH

LCALL	CMD

LCALL	WAIT_LCD

LCALL	UP_DOWN

MOV	RAM3,A		;D_2

MOV	A,#8EH

LCALL	CMD

LCALL	WAIT_LCD

LCALL	UP_DOWN

MOV	RAM4,A		;D_2

MOV	A,#0C5H

LCALL	CMD

LCALL	WAIT_LCD

LCALL	UP_DOWN

MOV	RAM5,A		;D_3

MOV	A,#0C6H

LCALL	CMD

LCALL	WAIT_LCD

LCALL	UP_DOWN

MOV	RAM6,A		;D_3

MOV	A,#0CDH

LCALL	CMD

LCALL	WAIT_LCD

LCALL	UP_DOWN

MOV	RAM7,A		;D_4

MOV	A,#0CEH

LCALL	CMD

LCALL	WAIT_LCD

LCALL	UP_DOWN

MOV	RAM8,A

MOV	P2,#0FFH

H3:		JB	RUN,G4

SJMP	H3

G4:		LCALL	DELAY

JB	RUN,RU1

SJMP	H3

RU1:		MOV	A,RAM1

SWAP	A

ANL	A,#0F0H

MOV	B,A

MOV	A,RAM2

CLR	C

SUBB	A,#30H

ADD	A,B

MOV	PACK1,A

MOV	A,RAM3

SWAP	A

ANL	A,#0F0H

MOV	B,A

MOV	A,RAM4

CLR	C

SUBB	A,#30H

ADD	A,B

MOV	PACK2,A

MOV	A,RAM5

SWAP	A

ANL	A,#0F0H

MOV	B,A

MOV	A,RAM6

CLR	C

SUBB	A,#30H

ADD	A,B

MOV	PACK3,A

MOV	A,RAM7

SWAP	A

ANL	A,#0F0H

MOV	B,A

MOV	A,RAM8

CLR	C

SUBB	A,#30H

ADD	A,B

MOV	PACK4,A

MOV	A,PACK1		;DECI TO HEX SUB..FOR R0

MOV	R4,A

LCALL	D_T_H

MOV	PACK1,A

MOV	A,PACK2		;FOR R1

MOV	R4,A

LCALL	D_T_H

MOV	PACK2,A

MOV	A,PACK3		;FOR R2

MOV	R4,A

LCALL	D_T_H

MOV	PACK3,A

MOV	A,PACK4		;FOR R3

MOV	R4,A

LCALL	D_T_H

MOV	PACK4,A

NOP

MOV	R0,#99

MOV	R6,#00H		;DEVICE CONTROL SUB

MOV	P0,#0FH		;All Devices On	p1=00

L1:		LCALL	TIMER

INC	R6

MOV	A,R6

CJNE	A,PACK1,D1

CLR	P0.0

MOV	A,#85H		;SET CURSOR TO L

LCALL	CMD

MOV	A,#24H

LCALL	WRITE_TEXT

MOV	A,#24H

LCALL	WRITE_TEXT

MOV	A,R6

D1:		CJNE	A,PACK2,D2

CLR	P0.1

MOV	A,#8DH		;SET CURSOR TO L

LCALL	CMD

MOV	A,#24H

LCALL	WRITE_TEXT

MOV	A,#24H

LCALL	WRITE_TEXT

MOV	A,R6

D2:		CJNE	A,PACK3,D3

CLR	P0.2

MOV	A,#0C5H		;SET CURSOR TO L

LCALL	CMD

MOV	A,#24H

LCALL	WRITE_TEXT

MOV	A,#24H

LCALL	WRITE_TEXT

MOV	A,R6

D3:		CJNE	A,PACK4,D4

CLR	P0.3

MOV	A,#0CDH		;SET CURSOR TO L=1,P=5

LCALL	CMD

MOV	A,#24H

LCALL	WRITE_TEXT

MOV	A,#24H

LCALL	WRITE_TEXT

MOV	A,R6

D4:		DJNZ	R0,L1

MoV	P0,#00H

MOV	P0,#1FH

LCALL	CLEAR_LCD

MOV	A,#82H		;set cursor to frist position.

LCALL	CMD

MOV	A,#'T'

LCALL	WRITE_TEXT

MOV	A,#'H'

LCALL	WRITE_TEXT

MOV	A,#'A'

LCALL	WRITE_TEXT

MOV	A,#'N'

LCALL	WRITE_TEXT

MOV	A,#'K'

LCALL	WRITE_TEXT

MOV	A,#' '

LCALL	WRITE_TEXT

MOV	A,#' '

LCALL	WRITE_TEXT

MOV	A,#'Y'

LCALL	WRITE_TEXT

MOV	A,#'O'

LCALL	WRITE_TEXT

MOV	A,#'U'

LCALL	WRITE_TEXT

MOV	A,#0CH

LCALL	CMD

LJMP	EXIT

INIT_LCD:	MOV	A,#38H		;LCD Initialization

LCALL	CMD		;2 lines 5x7 matrix

LCALL	CLEAR_LCD

MOV	A,#0EH

LCALL	CMD		;Display On,Cursor On

MOV	A,#06H

LCALL	CMD		;Increment Cursor

RET

CMD:		LCALL	WAIT_LCD

CLR	RS

MOV	DATA1,A

CLR	RW

SETB	EN

LCALL	DELAY

CLR	EN

RET

WAIT_LCD:	SETB	DB7		;P1.7 AS INPUT

CLR	RS		;RS=0

SETB	RW		;R/W=1 TO READ

BACK:		CLR	EN		;E=0

LCALL	DELAY

SETB	EN		;E=1

JB	DB7,BACK	;STAY UNTIL BUSY FLAG=0

RET

DELAY:		MOV	R2,#37H

AGAIN:		MOV	R3,#225H

HERE1:		NOP

NOP

DJNZ	R3,HERE1

DJNZ	R2,AGAIN

RET

CLEAR_LCD:	MOV	A,#01H		;Clear LCD Sub-routine

LCALL	CMD		;Clear LCD

RET

WRITE_TEXT:	LCALL	WAIT_LCD

SETB 	RS		;Data Write On LCD Sub-Routine

CLR	RW

MOV	DATA1,A

SETB	EN

LCALL	DELAY

CLR	EN

RET

UP_DOWN:	NOP

S2:		MOV	A,#'0'

S1:		LCALL	WRITE_TEXT

LCALL	DELAY

LO1:		JB	UP,G3

JB	DOWN,G3

JB	ENT,G3

SJMP	LO1

G3:		LCALL	DELAY

LCALL	DELAY

LCALL	DELAY

LCALL	DELAY

LCALL	DELAY

LCALL	DELAY

LCALL	DELAY

LCALL	DELAY

LCALL	DELAY

LCALL	DELAY

NOP

JB	UP,IN1

JB	DOWN,DE1

JB	ENT,ST1

SJMP	LO1

IN1:		MOV	R5,A		;MOV ACC. TO R5

MOV	A,#10H

LCALL	CMD		;MOVE CURSOR LEFT

MOV	A,R5

INC	A

CJNE	A,#3AH,S1

SJMP	S2

DE1:		MOV	R5,A

MOV	A,#10H

LCALL	CMD

MOV	A,R5

DEC	A

CJNE	A,#2FH,S1

SJMP	S2

ST1:		RET

D_T_H:		ANL	A,#0F0H

SWAP	A

MOV	R5,A

JZ	NOCHANGE

MOV	A,#00H

AG:		ADD	A,#06H

DJNZ	R5,AG

MOV	R6,A

MOV	A,R4

CLR	C

SUBB	A,R6

RET

NOCHANGE:	MOV	A,R4

RET

TIMER:		MOV	R7,#14H		;Timer time=50ms and loop=14H=20D

MOV	TMOD,#10H	;Timer 0 mode 1(16 - bit)

REP:		MOV	TL1,#0FEH	;TL0=4b, Low byte

MOV	TH1,#4BH

SETB	TR1		;Start Timer 0

AGA:		JNB	TF1,AGA

CLR	TR1		;Stop	Timer 0

CLR	TF1		;Clear Timer 0 flag for next round

DJNZ	R7,REP		;Jump for Loop

RET

EXIT:		NOP

SJMP	EXIT

NOP

END

COMPONENTS

ENCODER

In digital circuits, the term 'multiplexing' is also sometimes used to refer to the process of encoding, which is basically the generation of a digital code to indicate which of several input lines is active.  An encoder or multiplexer is therefore a digital IC that outputs a digital code based on which of its several digital inputs is enabled.

On the other hand, the term 'demultiplexing' in digital electronics is also used to refer to 'decoding', which is the process of activating one of several mutually-exclusive output lines, based on the digital code present at the binary-weighted inputs of the decoding circuit, or decoder. A decoder or demultiplexer is therefore a digital IC that accepts a digital code consisting of two or more bits at its inputs, and activates or enables one of its several digital output lines depending on the value of the code.

Multiplexing and demultiplexing are used in digital electronics to allow several chips to share common signal buses. In demultiplexers, for instance, the output lines may be used to enable memory chips that share a common data bus, ensuring that only one memory chip is enabled at a time in order to prevent data clashes between the chips.

DECODER

A decoder is a device which does the reverse of an encoder, undoing the encoding so that the original information can be retrieved. The same method used to encode is usually just reversed in order to decode.

In digital electronics, a decoder can take the form of a multiple-input, multiple-output logic circuit that converts coded inputs into coded outputs, where the input and output codes are different. e.g. n-to-2n, binary-coded decimal decoders. Enable inputs must be on for the decoder to function, otherwise its outputs assume a single "disabled" output code word. Decoding is necessary in applications such as data multiplexing, 7 segment display and memory address decoding.

The example decoder circuit would be an AND gate because the output of an AND gate is "High" (1) only when all its inputs are "High." Such output is called as "active High output". If instead of AND gate, the NAND gate is connected the output will be "Low" (0) only when all its inputs are "High". Such output is called as "active low output".

Fig:- A 2-to-4 Line Single Bit Decoder

A slightly more complex decoder would be the n-to-2n type binary decoders. These types of decoders are combinational circuits that convert binary information from 'n' coded inputs to a maximum of 2n unique outputs. We say a maximum of 2n outputs because in case the 'n' bit coded information has unused bit combinations, the decoder may have less than 2n outputs. We can have 2-to-4 decoder, 3-to-8 decoder or 4-to-16 decoder. We can form a 3-to-8 decoder from two 2-to-4 decoders (with enable signals).

Similarly, we can also form a 4-to-16 decoder by combining two 3-to-8 decoders. In this type of circuit design, the enable inputs of both 3-to-8 decoders originate from a 4th input, which acts as a selector between the two 3-to-8 decoders. This allows the 4th input to enable either the top or bottom decoder, which produces outputs of D (0) through D (7) for the first decoder, and D (8) through D (15) for the second decoder.

A decoder that contains enable inputs is also known as a decoder-demultiplexer. Thus, we have a 4-to-16 decoder produced by adding a 4th input shared among both decoders, producing 16 outputs.

MULTIPLEXER

Multiplexing is defined as the process of feeding several independent signals to a common load, one at a time.  The device or switching circuitry used to select and connect one of these several signals to the load at any one time is known as a multiplexer.

The reverse function of multiplexing, known as demultiplexing, pertains to the process of feeding several independent loads with signals coming from a common signal source, one at a time. A device used for demultiplexing is known as a demultiplexer.

Multiplexing and demultiplexing, therefore, allow the efficient use of common circuits to feed a common load  with signals from several signal sources, and to feed several loads from a single, common signal source, respectively.

In digital circuits, the term 'multiplexing' is also sometimes used to refer to the process of encoding, which is basically the generation of a digital code to indicate which of several input lines is active.  An encoder or multiplexer is therefore a digital IC that outputs a digital code based on which of its several digital inputs is enabled.

On the other hand, the term 'demultiplexing' in digital electronics is also used to refer to 'decoding', which is the process of activating one of several mutually-exclusive output lines, based on the digital code present at the binary-weighted inputs of the decoding circuit, or decoder. A decoder or demultiplexer is therefore a digital IC that accepts a digital code consisting of two or more bits at its inputs, and activates or enables one of its several digital output lines depending on the value of the code.

Multiplexing and demultiplexing are used in digital electronics to allow several chips to share common signal buses. In demultiplexers, for instance, the output lines may be used to enable memory chips that share a common data bus, ensuring that only one memory chip is enabled at a time in order to prevent data clashes between the chips.

If a demultiplexer or decoder has 2N output lines, then it has N input lines. A common example of a decoder/demultiplexer IC is the 74LS138, which is a Low-Power Schottky TTL device that has 3 input lines and 8 output lines.  Of course, a decoder IC such as the 74LS138 also has chip control lines that need to be 'enabled' for the decoding function to take place.

LCD DISPLAY

This is a high quality 16 character by 2 line intelligent display module, with back lighting, Works with almost any microcontroller.

#### Features

- 16 Characters x 2 Lines

- 5x7 Dot Matrix Character + Cursor

- HD44780 Equivalent LCD Controller/driver Built-In

- 4-bit or 8-bit MPU Interface

- Standard Type

- Works with almost any Microcontroller

- Great Value Pricing

ULN2003

The ULN2001A, ULN2002A, ULN2003 and ULN2004A are high voltage, high current darlington arrays each containing seven open collector darlington pairs with common emitters. Each channel rated at 500mA and can withstand peak currents of 600mA. Suppression diodes are included for inductive load driving and the inputs are pinned opposite the outputs to simplify board layout These versatile devices are useful for driving a wide range of loads including solenoids, relays DC motors, LED displays filament lamps, thermal printheads and high power buffers.The ULN2001A/2002A/2003A and 2004A are supplied in 16 pin plastic DIP packages with a copper leadframe to reduce thermal resistance. They are available also in small outline package (SO-16) as ULN2001D/2002D/2003D/2004D.

PIN CONNECTION