HIGH PERFORMANCE ZIGBEE PROTOCOL IMPLEMENTATION FOR RF COMMUNICATION (STAR TOPOLOGY)

ZigBee is a wireless network protocol specifically designed for low rate sensor and control networks. Compared to other wireless protocols, the ZigBee wireless protocol offers

- low complexity,

- reduced resource requirements

- And most importantly, a standard set of specifications.

- It also offers three frequency bands of operation along with a number of network configurations

And optional security capability

The XBee/XBee-PRO ZB OEM RF Modules are designed to operate within the ZigBee protocol and support the unique needs of low-cost, low-power wireless sensor networks. The modules require minimal power and provide reliable delivery of data between remote devices. The modules operate within the ISM 2.4 GHz frequency band

ZigBee networks are called personal area networks or PANs. Each network is defined with a unique PAN identifier (PAN ID). XBee ZB supports both a 64-bit (extended) PAN ID and a 16-bit PAN ID. The 16-bit PAN ID is used in all data transmissions. The 64-bit PAN ID is used during joining, and to resolve 16-bit PAN ID conflicts that may occur.

In ZigBee networks, the coordinator must select a PAN ID (64-bit and 16-bit) and channel to start a network. After that, it behaves essentially like a router. The coordinator and routers can allow other devices to join the network and can route data. After an end device joins a router or coordinator, it must be able to transmit or receive RF data through that router or coordinator. The router or coordinator that allowed an end device to join becomes the "parent" of the end device. Since the end device can sleep, the parent must be able to buffer or retain incoming data packets destined for the end device until the end device is able to wake and receive the data.

High performance ZigBee network protocol implementation for RF communication projects main idea is to  develop a application which use Radio frequency to communication between network and Zig Bee is the network which will maintain the personal area network. In this network each network has its own unique PAN id. ZBee works on 64bit and 16 bit systems. Data transmission is done with 16 bit and 64 bit is used to solve any issues related to 16 bit.

Here we are following the star topology that contains one coordinator and no. of roots or end devices. Each device in a ZigBee network is having a different id. The coordinator collects the all devices id and always able to connect with the each device in its network. If a device is an out of range from the network then the coordinator builds the root to the destination by identifying the network id and broadcasts the data to the destination.