PC BASED DATA ACQUISITION SYSTEM BY STIMULATING SPI AND I2C PROTOCOL IMPLEMENTATION

Many times it is required to monitor physical phenomena such as temperature, pressure, humidity, light intensity, sound intensity, force, etc. Such physical phenomena can be monitored by digital systems employing sensors. However sensors generally produce a change in resistance, voltage or current. All such phenomena can be converted into a change in voltage levels. Since the sensors and the subsequent signal conditioning circuits provide a proportionate change in voltage with respect to the phenomena, this cannot be directly interpreted by a digital system. A sensor is usually an analog voltage and can take any value between + 10 to -10 volts. However a digital signal has only two states. 1 (ON) or 0 (OFF). Thus it is required to have additional circuitry between the digital system and analog system that translates the analog signal in to a digital signal. Such conversion can be performed by an analog to digital converter.

The Serial Peripheral Interface Bus or SPI (often pronounced "spy") bus is a synchronous serial data link standard that operates in full duplex mode. Devices communicate in master/slave mode where the master device initiates the data frame. Multiple slave devices are allowed with individual slave select (chip select) lines. The I2C bus physically consists of 2 active wires and a ground connection. The active wires, called SDA and SCL, are both bi-directional. SDA is the Serial Data line, and SCL is the Serial Clock line.

PC Based Data Acquisition System Using SPI and I2C protocol projects main idea is to implement sensor system which can monitor parameters like temperature, pressure, humidity, light intensity …etc. In general sensors are effected by  atmosphere changes but these changes can be converted in to change in voltage.

Data Acquisition refers to reading the data continuously from any sensor. Here we are continuously monitoring the data through PC. We are concentrating mostly on serial communication. The data which we are receiving from the sensor is received by the microcontroller using ADC. Data monitoring is done in PC and for communication serial port communication is used. We can get the information like Date and Time along with our data.

The data that is stored in the PC is easier to monitor or retrieve at any time compared to storing in a particular memory. Nowadays we are using the same process as storing the data in the PC in any kind of industries.